BUSINESS REPORT

Reducing Yarn Wastage in Weaving Section

Submitted To:

Mr. Kamal Hossain

Production Manager, Delta Fabrics Ltd.

Submitted By:

Quality Control & Production Team

Weaving Division, Delta Fabrics Ltd.

Date: March 01, 2026

Reference No: DFL/WD/2026/03

# 1. Executive Summary

This report investigates the problem of excessive yarn wastage in the Weaving Section of Delta Fabrics Ltd. during January–February 2026. The average wastage rate recorded was 8.4%, significantly above the industry benchmark of 4–5%. The report identifies the primary causes of wastage, analyzes their impact on production costs, and recommends corrective measures. If implemented, the proposed recommendations are projected to reduce wastage by approximately 50% within three months, saving an estimated BDT 3.2 lakh per month.

# 2. Introduction

## 2.1 Background

Delta Fabrics Ltd. is a medium-scale textile manufacturing unit located in Gazipur, Bangladesh. The Weaving Division operates 48 power looms producing cotton and polyester-cotton blend fabrics for both domestic and export markets. Yarn is the single largest raw material cost, accounting for approximately 62% of total production expenses.

## 2.2 Purpose of the Report

The purpose of this report is to:

- Identify the causes of abnormally high yarn wastage in the Weaving Section.

- Quantify the financial impact of the wastage on the company.

- Recommend practical steps to bring wastage within acceptable limits.

## 2.3 Scope

This report covers the period from January 1 to February 28, 2026, and focuses exclusively on the Weaving Section. Data from other departments such as dyeing and finishing are outside the scope of this report.

## 2.4 Limitations

Data collection was limited to shift supervisors' daily logs and machine maintenance records. A formal time-motion study could not be conducted within the reporting period due to ongoing production schedules.

# 3. Methodology

The following data collection methods were used in preparing this report:

- Review of daily production logs and shift reports (January–February 2026).

- Physical measurement of yarn input vs. fabric output using weighing records.

- Interviews with 12 loom operators, 3 shift supervisors, and the maintenance technician.

- Comparison of wastage data against industry standard benchmarks (BTMA guidelines).

- Analysis of machine maintenance and breakdown records.

# 4. Findings

## 4.1 Wastage Rate Data

The following table shows the monthly yarn wastage recorded in the Weaving Section:

| Month | Yarn Input (kg) | Fabric Output (kg) | Wastage Rate (%) |
| --- | --- | --- | --- |
| January 2026 | 18,450 | 16,720 | 9.4% |
| February 2026 | 17,800 | 16,380 | 7.9% |
| Average (2 months) | 18,125 | 16,550 | 8.4% |
| Industry Benchmark | — | — | 4–5% |

Table 1: Yarn Wastage in the Weaving Section (Jan–Feb 2026)

## 4.2 Key Causes Identified

Based on investigation, the following causes were identified as the main contributors to yarn wastage:

- Machine Misalignment: 14 out of 48 looms showed tension irregularities, causing frequent yarn breakage and subsequent wastage during re-threading.

- Operator Error: Newly recruited operators (6 workers, hired in December 2025) showed a wastage rate 3.1% higher than experienced workers, indicating insufficient training.

- Substandard Yarn Lot: A consignment of 3,200 kg of yarn (Lot No. RW-2025-118) was found to have inconsistent twist per inch (TPI), leading to higher breakage rates.

- Lack of Wastage Monitoring: No formal real-time wastage tracking system was in place; wastage was calculated only at the end of each week.

## 4.3 Financial Impact

At the current average wastage rate of 8.4% versus the target of 4.5%, the company is losing approximately 693 kg of yarn per month. At an average yarn cost of BDT 480/kg, this represents a monthly financial loss of:

693 kg × BDT 480 = BDT 3,32,640 per month

# 5. Conclusion

The investigation confirms that yarn wastage in the Weaving Section is significantly above the acceptable industry standard of 4–5%. The primary contributors are mechanical issues with looms, insufficient operator training, a specific substandard yarn consignment, and the absence of a real-time monitoring system. Collectively, these factors are costing the company over BDT 3.3 lakh per month in unnecessary raw material losses. Timely and targeted corrective action is essential.

# 6. Recommendations

Based on the findings, the following actions are recommended:

- Immediate Machine Maintenance: Schedule servicing and realignment of all 48 looms within the next two weeks. Priority should be given to the 14 identified looms with tension problems.

- Operator Training Programme: Conduct a 3-day structured training session for the 6 newly recruited operators, covering loom operation, yarn handling, and wastage reduction techniques.

- Quarantine Substandard Yarn Lot: Immediately isolate Lot No. RW-2025-118 and negotiate with the supplier for replacement or credit. Future procurement should include mandatory TPI testing before acceptance.

- Implement Daily Wastage Tracking: Introduce a shift-wise wastage log system where supervisors record yarn input and fabric output at the end of each shift. Weekly summary reports should be submitted to the Production Manager.

- Set Departmental Wastage Targets: Establish a monthly wastage reduction target of 1% per month over the next four months, aiming to reach the 4.5% benchmark by July 2026.

# 7. References

- Bangladesh Textile Mills Association (BTMA) — Production Efficiency Guidelines, 2024 Edition.

- Delta Fabrics Ltd. — Daily Production Logs, January–February 2026.

- Delta Fabrics Ltd. — Machine Maintenance Records, Q4 2025 – Q1 2026.

- Interview records with shift supervisors and loom operators, February 2026.

Report prepared by the Quality Control & Production Team, Weaving Division

Delta Fabrics Ltd., Gazipur, Bangladesh | March 2026